# Pneumatic Actuator Test Stand

# Install, Run, & Tune the Controller

Evan Comiskey – MIT FID Lab – 2024/08 – comiskey@mit.edu

# Description:

This How-To serves as a guide for installing the Pneumatic Actuator Test Stand’s PID control software. It will walk you through the entire process, from downloading an IDE to code, all the way to fine tuning your solenoid valve and associated controller gains. This guide assumes you have a functional hardware and electronics set-up, featuring an Arduino Nano board with PWM capabilities. Please reach out if you feel any critical information to be missing from this guide.

Table of Contents:

Description:	1

1.	Installing PlatformIO	2

2.	Downloading the Codebase	5

3.	Setting up your Working Directory with PlatformIO	6

4.	Running the Controller	9

5.	Adjusting Settings & Tuning Your Solenoid	11

6.	Processing Test Data	14

# Installing PlatformIO

- Use an Interactive Development Environment (IDE) capable of running PlatformIO projects.

  - Skip to section 2 if your IDE already supports PlatformIO projects.

- PlatformIO Overview:

  - PlatformIO is an open-source tool for working with embedded electronics systems like Arduino microcontrollers.

  - It offers a more in-depth and organized way to work with microcontroller code than the standard ArduinoIDE.

  - Can be run using various free and popular IDEs listed here.

- Setting up PlatformIO on Visual Studio Code (VS Code):

  - Download VS Code for Windows, Mac, or Linux at this link or by going to https://code.visualstudio.com/download.

  - Follow all default installation instructions, as PlatformIO does not require you to do anything specific during VS Code’s installation.

- Once you have successfully installed VS Code, you will see a screen as shown:

  - Feel free to go through VS Code’s provided tutorials as is necessary to familiarize yourself with the application.

- Install necessary C/C++ Dependencies

  - click the Extensions tab shown in the menu bar near the top left of the screen, highlighted in red below at left.

  - Type “C” into search bar. This will cause C/C++ by Microsoft to appear. This is the language which PlatformIO depends upon, so we must download it beforehand.

  - Click the install button for the extension.

- Similarly search for PlatformIO in the search bar. Download the first option that appears in the dropdown.

  - The author of this extension will also be named PlatformIO.

  - If you click on the extension, you should see a preview page which looks like the image shown at right.

  - It will take a few minutes for PlatformIO to fully installed. Once the application is fully installed, you will be prompted to quickly restart VS Code

- You will see a PlatformIO alien-head icon appear in the righthand menu where the extensions tab was once the application has successfully been installed.

  - Clicking on this icon will bring up the PlatformIO project tasks and quick access menus.

  - Press the PIO Home Open button in the quick access menus to bring up the PlatformIO homepage.

- You can start brand new projects from the PlatformIO homepage.

- PlatformIO is successfully installed if you see the home screen shown below at right, and you are ready to start working with this project’s codebase.

# Downloading the Codebase

- Download the codebase by accessing the GitHub repository associated with this project by clicking here, or by going to https://github.com/Comiskey/soft-actuator-controller.

  - If this repo is private and inaccessible, contact Evan Comiskey

- Once here, click the green Code dropdown button, then click Download ZIP

- Unpack the contents of this zip file in an easily accessible file location.

  - Note that at this point, you should not interact with the code directly. The first time you open this folder in a code editor, you should do so through PlatformIO, following the steps explained below.

# Setting up your Working Directory with PlatformIO

- Navigate to the PlatformIO home page and click the Import Arduino Project button

- An import menu will appear.

  - This menu has two parts: board selection and folder directory selection.

  - Click board selection, and type Nano into the search bar. From here,

    - select Arduino Nano ATmega328 (New Bootloader).

  - Click the Use Libraries installed by Arduino IDE checkbox to include necessary Arduino dependencies for the codebase.

  - In folder directory selection, navigate to where you had previously unpacked the downloaded GitHub repository, then open the controller sub-folder. You should see the folder structure highlighted in blue in the directory selection sub-screen below. Once you do, click the Import button.

- You will have to redo this process if you decide to move the repository within your computer’s folder structure later, as PlatformIO will have to reinstall its dependencies in that new location.

- PlatformIO will begin automatically formatting the content of the directory to match the selected board once the project has been imported.

  - This may take a few minutes.

  - You can also change this later in the platform.ini file.

- Set-up will take a few seconds, but once completed, will bring you to the following screen:

- PlatformIO has opened the controller code’s directory in VS Code’s Explorer tab (highlighted in red above).

  - From now on, whenever you open the folder in VS Code, all necessary PlatformIO dependencies will open as well.

- You will know that PlatformIO is properly configured and running by the series of button options which will appear at the bottom of the screen when the directory is opened:

- Of these buttons, the most important are the following:

  - Home: Brings up the PlatformIO home page shown previously at the top of section 3

  - Build: Compiles all code in the directory. This button allows you to check the code for syntax errors without having to be directly connected to a microcontroller

  - Upload: Compiles, and sends the code to the microcontroller

  - Serial Monitor: Opens the serial monitor. Useful for set-up and general debugging

- Press the Build button to ensure that the directory has been properly configured as a PlatformIO project.

  - If it has been, the terminal will automatically open and present a success message after a few seconds as shown:

- Move on to running the controller once you see the success message appear in the terminal.

# Running the Controller

- This section will cover how to run the controller during normal operation/cycle testing.

  - This section assumes that your solenoid valve has been properly tuned prior to your use of the stand. If this is not the case, please read through this section for context, before moving on to section 5 to be walked through controller tuning procedures.

  - If someone has in your group has successfully used the stand before, you do not need to read section 5.

- All settings relevant to the operation of the controller are found in a single file: adjustableSettings.cpp, which lives in the src folder.

  - adjustableSettings.cpp only contains variable definitions and is only relevant to quick set-up, adjustment, and use of the controller.

  - This section will only reference variables without in-depth explanations

  - For a detailed explanation of what each variable does, please see the Test Stand & Controller Overview Word document.

- Ensure that the first 10 variables—pin definitions—match your electronic’s layout.

- Ensure that both TUNE_PRESSURE and TUNE_VENT are set to false.

  - Otherwise, the controller will operate in tuning mode, rather than as the normal controller.

- The next relevant setting will be USE_KPA.

  - If true, it will record data in kilopascals.

  - If false, it will record in PSI.

  - Trajectory setpoints, as well as the overpressure limit, will automatically be interpreted in the units associated with this variable.

- Next, select your frequencies for pressure reading/writing, interpolation, and controller action.

- Then, set your PID gain values with KP, KI, and KD (usually left as 0).

- Set the error THRESHOLD for the trajectory.

  - The smaller this value, the less error between the trajectory setpoint and the measured setpoint it will take to trigger the “failure to follow trajectory” test stop condition.

- Once settings are adjusted to your liking:

  - plug your computer into the USB port of the Arduino Nano on the electronics stand

  - press the upload button highlighted in section 3, wait for the terminal to display the success message

  - unplug your computer from the microcontroller

  - plug your electronics board into a 24V DC power supply

  - connect your pressure source to the inlet pressure line.

  - Plug your microSD card into the appropriate slot on the board.

  - connect your volume line up to your actuator or volume

  - turn the power to the board on.

- Once these steps are completed, the system will open the venting valve and close the inlet pressure for 5 seconds to ensure the actuator is not unintentionally overinflated.

- Next, it will initialize the SD card.

  - During this initialization, a csv file is generated with the format of cycle_test_#.csv, where # is the first available lowest number not yet stored on the SD card. This file will contain all the data during that test.

    - If tests 1-9 and 11-12 are on the card, 10 will be written

  - If the Arduino is reset, or power to the board is turned off/lost at any point, the controller will consider this a new test, and begin writing to a new file upon start-up/reset.

  - Your specified settings remain stored on the microcontroller until the next upload, so you will only have to reupload code to the computer in the event you want to make new adjust any settings.

- Once the SD card has been initialized, the LCD on the electronics board will prompt you to press the start button to begin the test and will wait indefinitely for you to do so.

- After the start button is pressed, the test will begin.

- During the test, the total number of completed cycles of your specified trajectory will be displayed on the LCD, along with the file on the SD card that data for this test is being written to. This is shown at right.

- Write down the name of this test file somewhere you will remember, along with your associated test conditions.

- When the test ends (stop pressed, failure to follow trajectory, or overpressure) the reason for test stoppage will replace the test file name on screen, as shown at right.

- Once a test has ended, power the board off with the switch.

- In all test-ending scenarios, the inlet valve is closed and then the venting valve is opened inside the solenoid.

  - The inside of the actuator volume is always reset to atmospheric pressure and is safe to remove from the pneumatic system.

- Disconnect/close your pressure source.

- Remove the SD card from its port to extract the data stored in the csv files onto your personal computer to process as you see fit.

  - So long as memory is available on the card, the controller program will never overwrite existing data on an SD card. Feel free to perform multiple tests before ever extracting data from the SD card.

# Adjusting Settings & Tuning Your Solenoid

- This section will explain how to tune your controller to the specific solenoid valve used in your set-up.

- Even if you are using the same model of Clippard electronic valve listed in the BOM, you must still tune the controller to that specific 2/2 solenoid valve, as all solenoid valves physically vary to a slight degree.

  - If someone else in your group has used this system for tests before, you do not need to go through this tuning procedure, and just use the controller normally.

- To get started, set TUNE_PRESSURE to true as shown, and upload the code to the controller.

  - Ensure the end of your line to test is capped, such as with a closed valve.

- Power the system on before opening the inlet airline to the system

- Open the serial monitor on your computer by clicking the button specified in section 3.

  - The open serial monitor will look something like what is shown below.

- Set the Baud rate option to 115200. Connect to the appropriate port on your computer using the port dropdown menu.

  - If you are familiar with ArduinoIDE, the PlatformIO’s serial monitor behaves in the exact same way for projects build on the Arduino libraries.

- Wait for the controller to go through its start-up routine; its steps displayed on the LCD on the electronics stand.

- Press the start button once prompted by the LCD display shown at left.

- You should now see pressure and PWM readings start to appear on the LCD.

  - If nothing is displaying, ensure you have pressed the start monitoring button, and are connect to the correct port on your personal device.

- With the pressure and PWM values being displayed at regular intervals in the serial monitor, now simply wait.

- At the point that the valve begins to open, you will see a change in the pressure reading on screen and hear a click or high-pitched whine coming from the solenoid.

- At this point, press the stop button on the electronics stand, but do not close or reset the serial monitor.

  - You will want to comb back through this pressure and PWM data to observe at what exact PWM value the valve began to open.

- Set ANALOG_PRESSURE_MIN to this value and save your change to the code.

- Repeat this entire process again, this time setting TUNE_PRESSURE false and TUNE_VENT to true.

  - As part of the vent valve tuning protocol, the volume line will fill for 0.1 seconds before holding both valves shut.

- You will then want to observe the serial monitor for decreases in pressure values as the PWM signal increases.

- Once you have determined which PWM value corresponds to the first noticeable decrease in pressure, set that value to ANALOG_VENT_MIN.

  - For a new Clippard EFB-1DV-24-L, expect the values to be in a nominal range between 120 and 200, and the mins and maxes to be roughly 30-40 values apart from one another.

- Now define the end of the corresponding ranges, also known as ANALOG_PRESSURE_MAX and ANALOG_VENT_MAX.

  - Based on prior testing, it is recommended to initially set both these values at their respective MIN value + 30 (i.e. 180, if your min value is 150).

- Once set, start tuning the controller gains to your specific actuator of choice using the normal operating procedures detailed in section 4.

  - If during this PID gain tuning, you find that the controller produces bang-bang control-like hysteresis around your setpoints, consider reducing the PWM range that can be written to the solenoids by reducing the values of either MAX variable as appropriate.

    - bang-bang-like behavior during periods of positively sloping trajectory should be met with decreases to ANALOG_PRESSURE_MAX, and bang-bang-like behavior during periods of negatively sloping trajectory should be met with decreases to ANALOG_VENT_MAX.

  - If you instead find that the controller is incapable of producing responses within what you consider a reasonable timeframe, consider increasing the PWM range that can be written to the solenoids by increasing the MAX values when appropriate following the same positive/negative slope checks as above.

# Processing Test Data

- This section explains how to get started processing your test data, using the included MATLAB processing scripts as a jumping off point for your own analysis.

- A test file from this controller contains time data in milliseconds, pressure, the controller error and integral values, and a cycle start marker organized as shown:

- Included in the repository for this controller are two MATLAB scripts: DebuggingDashboard.m and PressureVsTimeOverlap.m, along with a few helper functions.

- DebuggingDashboard.m displays the behavoir of the controller over time based on the specific Kp and Ki values used for a given test test to help determine optimal controller behavoir. It produces plots like shown:

  - PressureVsTimeOverlap.m displays all completed cycles from the test overlayed atop one another, using the cycle_start (normally 0) variable to deliniate the start and end (demarked as 1) of each cycle.

    - This script is helpful for quick data sharing, and for checking cycle-to-cycle consistency of the controller with your acutator

- Both helper funtions additionally plot an Ideal Trajectory reference function, which is plotted by defining the trajectory function in the getTracjectory.m helper function in the helper functions folder, in the same way as defined in the controller

  - See Write Custom Trajectory Functions for more information

- Additionally, there is a digital low-pass filter functionality included to allow for curve smoothing for creation of prettier graphs

  - This feature is set to off/false by default